INSPECTION & TEST PLAN

CONCRETE BORED PILES

DOC NO:

| 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 0 |  |  |  |  |  |
| Revision | Date | Written by | Checked by | Approved by | Remarks |

| Revision | Revision History |
| --- | --- |
| 0 | First issue |

|  |  |  | Project | (PROJECT NAME) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INSPECTION & TEST PLAN CONCRETE BORED PILES |  |  |  |  |  |  |  |
| Prepared by |  | Date |  | ITP No. | QA-XX-YYY- | Rev. No. | 0 |
| LEGEND: CON CONTRACTOR / IP = INSPECTION PARTY / CL = CLIENT R = REVIEW / S = SURVEILLANCE W=WITNESS / H=HOLD POINT |  |  |  |  |  |  |  |  |

| # | Submission / Inspection / Test details | Method | Inspection |  |  | Frequency | Approval By | Acceptance Criteria | Verification Record (register, inspection, test) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CON | IR | IP |  |  |  |  |
| 1 | DOCUMENTATION REVIEW |  |  |  |  |  |  |  |  |
| 1a | Shop drawings & Detailed Section & Elevation Drawings | Review |  |  |  | 2-4 Weeks Prior to commencing of works | PM or CM | In accordance to contract specs, Design requirement, drawings | Contractor’s documentation Manufactures test certificates Lab Test Results |
| 1b | RMC supplier or batching plant details | Review |  |  |  |  | PM or CM | Approved supplier (if Contractor’s batching plant is used, refer to specific batching plant ITP) |  |
| 1c | Concrete mix design | Review |  |  |  |  | PM/CM/QE | As per contract specs |  |
| 1d | Trial mix approval | Review |  |  |  |  | PM/CM/QE | As per contract specs/ drawings, and ACT QC guidelines & schedule Valid calibration certificates |  |
| 1e | Steel manufacturer details | Review |  |  |  |  | PM/CM/QE | From approved supplier |  |
| 1f | Bentonite supplier details and chemical composition | Review |  |  |  |  | PM/CM/QE | Approved supplier and product as per contract specs |  |
| 1g | Method statement | Visual/Review of results |  |  |  |  | PM/CM/QE | As per contract specs/ drawings, and ACT QC guidelines & schedule |  |
| 2 | INSPECTION OF INCOMING MATERIALS: |  |  |  |  |  |  |  |  |
| 2a | Storage of material | Visual |  |  |  | All | QE | All materials to be stored as per project spec or standard | Material delivery check sheet |
| 2b | Bentonite | Third Party Lab Test |  |  |  | 1 sample for every 30MT delivered | QE | IS Code 2911-I-sect 2-apx A Density = 1.12 g/ml. Viscosity = 37 sec.pH ≤ 11.5 | Material Deliveries check sheet CHECK LIST |

| # | Submission / Inspection / Test details | Method | Inspection |  |  |  | Frequency |  | Approval By | Acceptance Criteria |  | Verification Record (register, inspection, test) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CON |  | IR | IP |  |  |  |  |  |  |
| 2c | Cement, Aggregates, Steel, Admixtures, ready mix concrete |  |  |  |  |  | Refer to ITP for Civil Work Materials and Batching plant (QA-ACT-PM-CWM-R2 ) |  |  |  |  |  |
| 3 | PRE-BORING INSPECTION |  |  |  |  |  |  |  |  |  |  |  |
| 3a | Check on pile location (peg positioning) and pile diameter | Visual / Mmnt |  |  |  |  | All | QE |  | As per approved drawings Max Deviation ≤ 75mm and ≤ 50mm for single piles for column |  | Inspn: record CHECKLIST |
| 3b | Rig positioning | Visual / Mmnt |  |  |  |  | All | QE |  | Center alignment with peg |  |  |
| 3c | Rig verticality (1) | Visual / Mmnt |  |  |  |  | All | QE |  | Deviation / rig height < 1.5% |  |  |
| 3d | Temporary casing installation | Visual / Mmnt |  |  |  |  | All | QE |  | Temporary casing to be provided for the full length of the pile wherever possible or to be extend at least 2m into non-collapsible stratum. |  |  |
| 3e | Check on joints of temporary casing and tremie pipes | Visual / Mmnt |  |  |  |  | All | QE |  | All joints to be water tight and the inner surface of the casing to be smooth and free of all internal projections. |  |  |
| 3f | Chisel diameter and clean bentonite ports | Visual / Mmnt |  |  |  |  | All | QE |  | Chisel Ø = Pile Ø - 75mm |  |  |
| 3g | Rig verticality (2) | Visual / Mmnt |  |  |  |  | All | QE |  | Deviation / rig height < 1.5% |  |  |
| 3h | Slurry pump readiness | Visual / Mmnt |  |  |  |  | All | QE |  | Pressure exerted by the drilling mud is always greater than the pressures exerted by the soils and external ground water. |  |  |
| 3i | Check for the level of bentonite | Visual / Mmnt |  |  |  |  | All | QE |  | Level of the drilling mud shall be maintained at least 1m above the level of external ground water |  |  |
| 3j | Bentonite specific gravity test | Visual / Mmnt |  |  |  |  | All | QE |  | ≤1.2 |  |  |
| 4 | BORING IN-PROGRESS |  |  |  |  |  |  |  |  |  |  |  |
| 4a | Bentonite specific gravity test | Visual |  |  |  |  | All | QE |  |  | To be checked every hour to confirm specific gravity is maintained as required. | Inspn: record CHECKLIST |
| 4b | Rig Verticality | Visual |  |  |  |  | All | QE |  |  | To be checked every 10m to ensure verticality is within specified tolerance. |  |
| 5 | POST-BORING |  |  |  |  |  |  |  |  |  |  |  |
| 5a | Bottom flushing | Visual / Mmnt |  |  |  |  | All | QE |  |  | Bottom to be cleaned properly Until bentonite density = 1.2 (IS Code 2911 part 2 p.22) | Inspn: record CHECKLIST |

| # | Submission / Inspection / Test details | Method | Inspection |  |  | Frequency |  | Approval By | Acceptance Criteria | Verification Record (register, inspection, test) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CON | IR | IP |  |  |  |  |  |
| 5b | Borehole depth | Visual / Mmnt |  |  |  | All | QE |  | As per drawing | Inspn: record CHECKLIST |
| 5c | Spillage of bentonite | Visual |  |  |  | All | QE |  | Prevent spillage in areas outside boring area/ remove discarded slurry immediately |  |
| 6 | RE-BAR CAGE & TREMIE PIPE INSTALLATION |  |  |  |  |  |  |  |  |  |
| 6a | Check for rebar cage installation | Visual |  |  |  | All | QE |  | As per approved drawing | QA-ACT-PM-ISP-CBP-RBC-_____ Inspn: record CHECKLIST |
| 6b | Tremie pipe installation | Visual |  |  |  | All | QE |  | Bottom of tremie to toe level = Ø of pile +/-5% Spot measurements to be taken at suitable intervals to check that the tremie pipe has an adequate penetration into previous concrete. |  |
| 7 | PRE-POUR (NOT LATER THAN 12 HOURS AFTER LAST BOTTOM FLUSHING) |  |  |  |  |  |  |  |  |  |  |  |  |
| 7a | Bentonite gravity test | Visual |  |  |  | All | QE |  | Density ~ 1.2 | Inspn: record CHECKLIST |
| 7b | Check on Concrete mix | Visual |  |  |  | All | QE |  | Logged, RMC grade confirmed and slump test done and cubes casted as per requirement |  |
| 8 | CONCRETE POURING |  |  |  |  |  |  |  |  |  |
| 8a | Check on first charge of concrete | Visual / Mmnt |  |  |  | All | QE |  | The first charge of concrete shall be placed in the hopper over a sliding plate of the bottom of the hopper | Inspn: record CHECKLIST |
| 8b | Lifting of tremie | Visual / Mmnt |  |  |  | All | QE |  | Lifting speed < 3m/15mins |  |
| 8c | Check on continuity of pour | Visual / Mmnt |  |  |  | All | QE |  | Concreting of the pile shall be in one single and continuous operation |  |
| 8d | Cut-off level | Visual / Mmnt |  |  |  | All | QE |  | Concrete shall be cast at least 500mm above the cut-off level for cut-offs up to 1.5m below ground level. For deeper cut-offs additional coverage of 150mm will be required for every metre of additional increase in the cut-off below ground level. |  |
| 9 | POST POUR |  |  |  |  |  |  |  |  |  |  |  |  |
| 9a | Check on finishing | Visual / Mmnt |  |  |  | All | QE |  | All loose and weak concrete to be chipped off | Inspn: record CHECKLIST |

| # | Submission / Inspection / Test details | Method | Inspection |  |  | Frequency |  | Approval By | Acceptance Criteria | Verification Record (register, inspection, test) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CON | IR | IP |  |  |  |  |  |
| 9b | As-Built Eccentricity | Visual | H | H |  | All | QE |  | Approval by Structural Consultant (Eccentricity ≤ 75mm for group piles and ≤ 50mm for single piles | Inspn: record CHECKLIST |
| 9c | Compressive strength test on cubes | Visual | H | H |  | All | QE |  | As per project spec: |  |
| 9d | Integrity Test | Visual / Mmnt | H | H |  | All | QE |  | As per project spec: |  |
| 9e | Load test | Visual/ Mmnt | H | H |  | All | QE |  | As per project spec: |  |
| 10 | NCR and Inspection after Rectification of Defects | Refer to NCR Procedure in case of non compliance of tender/contract conditions. |  |  |  |  |  |  |  | Non-Conformance Report |  |  |

|  |  |  |  | Project | Project Name |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INSPECTION RECORD CONCRETE BORED PILES |  |  |  |  |  |  |  |
| ITP | Bored piles | Inspection No. | CHECKLIST |  |  | Location |  |
| Prepared by |  |  | Applicable Drawing |  |  |  |  |

| # | Description |  | Result |  | Date | Sign |  | # |  | Description | Result |  | Date | Sign |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cont: | IP |  |  |  |  |  |  | Cont: | IP |
| 3 | PRE-BORING INSPECTION |  |  |  |  |  |  | 6 |  | RE-BAR CAGE & TREMIE PIPE INSTALLATION |  |  |  |  |  |
| 3a | Check on pile location and pile diameter |  |  |  |  |  |  | 6a |  | Cage installation check |  |  |  |  |  |
| 3b | Rig positioning |  |  |  |  |  |  | 6b |  | Tremie pipe installation |  |  |  |  |  |
| 3c | Rig verticality (1) |  |  |  |  |  |  | 7 |  | PRE-POUR (not later than 12 hours after last bottom flushing) |  |  |  |  |  |
| 3d | Temporary casing installation |  |  |  |  |  |  | 7a |  | Bentonite gravity test |  |  |  |  |  |
| 3e | Check on joints of temporary casing and tremie pipes |  |  |  |  |  |  | 7b |  | Check on Concrete mix |  |  |  |  |  |
| 3f | Chisel diameter and clean bentonite ports |  |  |  |  |  |  | 8 |  | CONCRETE POURING |  |  |  |  |  |
| 3g | Rig verticality (2) |  |  |  |  |  |  | 8a |  | Check on first charge of concrete |  |  |  |  |  |
| 3h | Slurry pump readiness |  |  |  |  |  |  | 8b |  | Lifting of tremie |  |  |  |  |  |
| 3i | Check for the level of bentonite |  |  |  |  |  |  | 8c |  | Check on continuity of pour |  |  |  |  |  |
| 3j | Bentonite specific gravity test |  |  |  |  |  |  | 8d |  | Cut-off level |  |  |  |  |  |
| 4 | BORING IN-PROGRESS |  |  |  |  |  |  | 9 |  | POST POUR |  |  |  |  |  |
| 4a | Bentonite specific gravity test |  |  |  |  |  |  | 9a |  | Check on finishing |  |  |  |  |  |
| 4b | Rig Verticality |  |  |  |  |  |  | 9b |  | As-Built Eccentricity |  |  |  |  |  |
| 5 | POST-BORING |  |  |  |  |  |  | 9c |  | Compressive strength test on cubes |  |  |  |  |  |
| 5a | Bottom flushing |  |  |  |  |  |  | 9d |  | Integrity Test |  |  |  |  |  |
| 5b | Borehole depth |  |  |  |  |  |  | 9e |  | Load test |  |  |  |  |  |
| 5c | Spillage of bentonite |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONCLUSION |  | Accepted Accepted with Comments Rejected NCR No. _______ |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONTRACTOR |  |  |  |  |  |  |  |  | INSPECTION PARTY |  |  |  |  |  |  |
| Sign |  |  |  | Date |  |  |  |  | Sign |  |  | Date |  |  |  |

|  |  |  |  | Project | (PROJECT NAME) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INSPECTION RECORD CONCRETE BORED PILE REBAR CAGE FABRICATION |  |  |  |  |  |  |  |
| ITP | Bored Piles | Inspection No. | QA- |  |  | Location |  |
| ACT QC Inspector |  |  | Applicable Drawing |  |  |  |  |

| # | Description | Method | Frequ ency | Acceptance Criteria | Result | Date | Sign |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Contractor | IP |
| 1 | Cage length & diameter | Tape, visual | All | As per approved drawings, details and BBS and standards |  |  |  |  |
| 2 | Vertical bars, length, spacing, lapping | Tape, visual | All |  |  |  |  |  |
| 3 | Splicing, welds | Visual | All |  |  |  |  |  |
| 4 | Stirrups/Ties, spacing, lapping | Tape, visual | All |  |  |  |  |  |
| 6 | Spacer blocks, ring supports, tie wires | Visual | All |  |  |  |  |  |
| 7 | Sonic tubes fixing (if applicable) | Visual | All |  |  |  |  |  |
| 8 | Lifting hook: diameter, width, length | Tape, visual | All |  |  |  |  |  |
| 10 | Cleanliness | Visual | All |  |  |  |  |  |

| CONCLUSION | Accepted Accepted with Comments Rejected NCR No. ____________ |  |  |  |
| --- | --- | --- | --- | --- |
| COMMENTS |  |  |  |  |
| CONTRACTOR |  |  | INSPECTION PARTY |  |
| Date |  | Sign | Date | Sign |